Nicolet FEAR Chain Tensioning Method

Problem: Our team has had trouble tensioning chain in the past on our robot so that the chain does not skip/break.

This is a method we developed November & December 2016.

Working Criteria for System:

- Works for multiple chain sizes & lengths

  - 25 & 35 chain

  - Lengths that would be on robot

- Ease of use

  - Under 15 min. of training

- Low cost

  - Less than $30

- 2 part system

  - Off-robot measuring technique

  - On-robot device

Research

| Reason for Research | Source | Information |
| --- | --- | --- |
| Products we could buy | https://www.mcmaster.com/#chain-tensioners/=15etjis | Bought idlers are VERY expensive |
| Techniques that exist | http://www.hardcoresledder.com/forums/350-mini-sled-forum/424039-chain-tensioner.html |  |
|  | https://www.chiefdelphi.com/media/photos/26057 |  |
|  | https://www.chiefdelphi.com/forums/showthread.php?t=61099 | Off-center hole in HDPE idler, rotate to tension |
|  | https://www.chiefdelphi.com/forums/showthread.php?t=102013 | PVC idler or sprocket idler |
|  | Shared PDF (“Chain Manufacturer's’ Manual”) in Chain Tensioning folder | Where idlers can be added |
| Companies that make idlers | http://www.botlanta.org/converters/dale-calc/sprocket.html |  |
|  | http://www.brewertensioner.com/products.html |  |
| General knowledge about robotics | http://hcwilson.weebly.com/uploads/3/8/4/6/38463501/frc_guide_-_arpan_rao.pdf | See link for information & tips on everything robotics from another FRC team |
| Chain length calculators | http://www.botlanta.org/converters/dale-calc/sprocket.html |  |
|  | http://www.islandpondrailroad.com/chain.htm |  |
| Plastic for idler | http://www.vanderveerplastics.com/compare-materials.html?sel1=abs-machined-parts&sel2=hdpe | ABS v. HDPE |
|  | https://en.wikipedia.org/wiki/Acrylonitrile_butadiene_styrene | ABS |
|  | http://www.makerfarm.com/index.php/hardware/delrin-wheels-idlers.html | Delrin |
|  | http://www.rccrawler.com/forum/tools-procedures/128834-cutting-delrin.html | How to cut delrin |
|  | https://www.mcmaster.com/#standard-acetal-homopolymer-rods/=15bavkq | McMaster - Carr: Delrin |
|  | https://www.mcmaster.com/#hdpe/=15bavur | McMaster - Carr: HDPE |
|  | https://www.mcmaster.com/#polyamide/=15bau0f | McMaster - Carr: Polyamide |
| How to break & add chain | http://www.hardcoresledder.com/forums/350-mini-sled-forum/424039-chain-tensioner.html | Video tutorial |

Adding Chain:

Use math equation

- http://www.botlanta.org/converters/dale-calc/sprocket.html

- http://www.islandpondrailroad.com/chain.htm

We used these equations to find the number of chain links for axles that are 10.5 in. apart. The length was 1-2 links off. Use the math equation as an approximation and add/subtract links as necessary.

Tensioning Chain:

Idea 1: Spring Loaded Idler

CAD & Drawing:

Pictures of Manufactured Part:

Materials:

From McMaster Carr:

- $5.48 / ½ ft. Low-carbon steel bar, 2.5in. / arm = $2.25 each

- $3.80 ea. ¼ in. 303 Stainless Steel Shaft Collar

- Spring still requires further research, but is estimated around $5 (Spring in prototype was taken from an old motor)

From Amazon:

- $4.76 / 5 pc Miniature ball bearing = $1 each

From Andymark:

- $16.00 ea. 22 tooth hubbed 25 sprocket

Process:

- Manufacture steel part with CNC or mill

- Boar out hole in sprocket with CNC or mill so that bearing can be press fit into it

- Press fit bearing into sprocket

- Put sprocket on arm of steel part and lock with shaft collar

- Put spring on and attach to frame

Idea 2: Plastic Delrin Idler

CAD & Drawing:

Pictures of Manufactured Part:

Materials:

From McMaster Carr:

- $10.37 Wear- and Water-Resistant Delrin® Acetal Resin (1.5 in. diameter, black), 1 in. per device

- ¼ 20 bolt and nut

Process:

- Cut delrin to 1 in. long with vertical band saw

- Drill hole with mill

Decision Matrix

| Criteria | Works for 25 & 35 chain | Would work for lengths that would be on robot | Ease of use (amount of time to train to use) | Cost per 1 device | Time to make | Time to install | Certifications required to make | Quality of tensioning | Totals |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| How to measure | Prototype | Prototype | Talk to someone not on chain tensioning team and see if they understand, time how long to explain 1 = more than 10 min. 2 = 3 - 10 min. 3 = up to 3 min. | Add costs of parts 1 = more than $10 2 = up to $10 3 = Materials we have | Time how long to make 1 = 20 min. Or more 2 = 10 - 20 min. 3 = 10 min. Or less | Time how long to install 1 = 20 min. Or more 2 = 10 - 20 min. 3 = 10 min. Or less | 1 = Advanced zip ties 2 = Intermediate zip ties 3 = Basic zip ties | Not able to test - would need prototype able to move (not enough time to test) |  |
| Weights: | 3 | 3 | 3 | 3 | 1 | 2 | 2 | 3 |  |
| Spring Loaded Idler | 3 (9) | 3 (9) | 3 (9) | 2 (6) | 1 (1) | 2 (4) | 1 (2) |  | 40 |
| Plastic Delrin Idler | 3 (9) | 3 (9) | 3 (9) | 2 (6) | 3 (3) | 3 (6) | 1 (2) |  | 44 |

1=bad 3=good

Conclusion:

- These designs have not been tested on a moving prototype, so more testing would need to be done before using them on a robot

- We can combine the designs into a delrin idler on the spring system